# Riverside test rig: guard interlock trips, September 2026

Between 1 and 26 September 2026 the guard interlock on the Riverside test rig tripped 41 times over 19 shifts, about twice a shift. Each trip stops the pump under test and costs 20 to 30 minutes of rig time.

Causes, from the rig book and the controller's event log: 29 trips came from guard door sensor GS-2 on the rear guard door; 8 came from operators opening the front guard during a run; 4 came from the PLC watchdog after the controller firmware was updated to 4.2 on 15 September.

GS-2: the sensor bracket on the rear guard door is cracked at the weld. The closed door sags about 3 mm, and above 1,500 rpm the rig's vibration moves the magnet past the sensor's 5 mm switching distance. The sensor itself tested good on the bench.

Recommendation 1: replace the bracket and fit a coded magnetic safety sensor (Sentrel CMS-40) in place of GS-2. Sentrel Automation quoted 3,400 pounds including fitting, valid until 31 October 2026. Adding a software debounce to the safety channel was considered and rejected by the safety review: a trip on the safety channel must never be delayed.

Recommendation 2: operator trips. Re-brief the rig team on the start-up checklist and fit a 'run in progress' beacon above the front guard (quoted at 620 pounds).

Recommendation 3: watchdog trips. Firmware 4.2 lengthened the controller's scan cycle. Raise it with the controls team before the Halden rigs take the same update.

Owner: Sam Reyes, automation engineering. Review with the Riverside service manager on 9 October 2026.